# ARTICLE XI: SPECIAL RESIDENTIAL REGULATIONS

# Article XX:

# RE: Amend Zoning Bylaw Article XI – Special Residential Regulations.  To see if the Town will vote to amend Article XI by adding the following regulations in blue as follows:

SECTION 11.1.0 DEVELOPMENT INCENTIVE FOR AFFORDABLE HOUSING

SECTION 11.2.0 ACCESSORY RESIDENTIAL USES IN ONE-FAMILY DWELLINGS: ACCESSORY APARTMENTS

SECTION 11.3.0 ADDITIONAL REGULATIONS AND EXCEPTIONS FOR GARDEN APARTMENTS AND DORMITORIES

SECTION 11.4.0 ADDITIONAL REGULATIONS FOR HOUSINGS FOR THE ELDERLY IN AN RO-ONE FAMILY RESIDENCE DISTRICT

SECTION 11.5.0 ADDITIONAL REGULATIONS IN THE CONTINUING CARE (RC) DISTRICT

SECTION 11.6.0 OPEN SPACE RESIDENTIAL DEVELOPMENT

SECTION 11.7.0   RESIDENTIAL 2 (R2) DISTRICT

SECTION 11.8.0 INCLUSIONARY ZONING REQUIREMENTS FOR MULTIFAMILY HOUSING

11.9. PERFORMANCE STANDARDS FOR RESIDENTIAL USES IN THE MIX AND MIXSD DISTRICTS

- Townhouse/Rowhouse.

- Approval Criteria.

- No more than eight (8) units may be attached by a common wall before an accessway of 20 feet is provided for pedestrians, vehicles, or outdoor amenity space.

- Off-street parking should not be allowed in front or between the buildings.  Parking should be located behind or to the side of the building and accessed by a public alley or private access street.

- The frontage area between the building façade and street line should be landscaped with grass or ornamental landscaping.

- Multi-Family - Attached Dwellings.

- Approval Criteria.

- Off-street parking should be located behind the front facade of a building facing a public street.

- Where there is a setback between the building façade and a public street line, there should be an accessible and usable open space such as a forecourt or plaza) with appropriate landscaping, streetscaping and furnishings.

- Mixed Use - Attached Dwellings.

- Approval Criteria.

- Mixed use buildings in the MIX district shall reserve the ground floor space for permitted non-residential uses to a depth of 60 feet from the primary street line.  Upper floors may be used by all permitted uses including residential uses.

- Off-street parking shall be located behind the front façade of a building facing a public street in the MIX district.

- Nursing Home, Assisted Living/Independent Living Facility, Continuing Care Retirement Community, Congregate Living Facility.

- Approval Criteria.

- Design considerations of the facility should consider an attractive walkable environment; an efficient access for emergency vehicles; visibility and design of the principal entrance; convenient resident drop-off area; usable outdoor amenity spaces; and communal indoor seating areas with a direct line of site to the outdoors.

- Hotel or Motel, Residence Hotel, Extended Stay Hotel or Motel.

- Approval Criteria.

- Compatibility with the level of activity associated with the surrounding properties.

- Capacity of the street and sidewalk network providing access to the site and impact on pedestrian, bicycle, and vehicular traffic and circulation patterns in the district.

- Location and visibility of the principal entrance, guest drop-off area, taxi queuing station, outdoor amenity space for guests or employees, and pedestrian circulation from all exit points.

- A hotel employee must be on-site at all times.

- A residence hotel/motel shall be located within 1,500 feet of Route 3 or Route 128/95.